🔬 EXPERIMENT 1: Interfacing LEDs / Switches with Embedded Core

📄 Source:

Aim

To interface LEDs and switches with an embedded controller and control LED operation based on switch input.

Apparatus Required

- Embedded board (Arduino UNO / 8051 / ARM)

- LED (Red/Green)

- Resistors (220Ω, 10kΩ)

- Push button switch

- Breadboard

- Connecting wires

- Power supply / USB

Theory

- Embedded systems use GPIO (General Purpose Input Output) pins to interact with hardware.

- LED → Output device

  - Glows when GPIO is HIGH

- Switch → Input device

  - Changes logic level when pressed

- Pull-up/pull-down resistors prevent floating input

Circuit Connections

Algorithm

- Start

- Set LED pin as OUTPUT

- Set switch pin as INPUT

- Read switch state

- If pressed → LED ON

- Else → LED OFF

- Repeat

Code

int buttonState = 0;

void setup()

{

pinMode(2, INPUT);

pinMode(LED_BUILTIN, OUTPUT);

}

void loop()

{

buttonState = digitalRead(2);

if (buttonState == HIGH) {

digitalWrite(LED_BUILTIN, HIGH);

} else {

digitalWrite(LED_BUILTIN, LOW);

}

delay(10);

}

Observation

| Switch | LED |
| --- | --- |
| Pressed | ON |
| Released | OFF |

Conclusion

LED and switch were successfully interfaced using GPIO. LED operation was controlled based on switch input, demonstrating basic digital I/O in embedded systems.

🔬 EXPERIMENT 2: Interfacing DHT11 Temperature & Humidity Sensor with Arduino

📄 Source:

Aim

To interface the DHT11 sensor with Arduino Uno and display temperature & humidity on Serial Monitor.

Apparatus Required

- Arduino Uno

- DHT11 Sensor

- 10kΩ resistor (if bare sensor)

- Breadboard

- Connecting wires

- USB cable

- Arduino IDE

Theory

- DHT11 measures:

  - Temperature

  - Humidity

- Contains:

  - Capacitive humidity sensor

  - Thermistor

  - Internal ADC

- Uses single-wire digital communication

Specifications

- Temp: 0°C to 50°C

- Humidity: 20–90% RH

- Accuracy: ±2°C, ±5% RH

Pin Configuration

| DHT11 Pin | Arduino |
| --- | --- |
| VCC | 5V |
| DATA | Pin 2 |
| GND | GND |

👉 Add 10kΩ pull-up between VCC & DATA (if needed)

Algorithm

- Start

- Initialize sensor & serial

- Read temperature & humidity

- Check if valid

- Display on serial monitor

- Repeat

Code

#include <DHT.h>

#define DHTPIN 2

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(9600);

dht.begin();

Serial.println("DHT11 Sensor Reading");

}

void loop() {

float humidity = dht.readHumidity();

float temperature = dht.readTemperature();

if (isnan(humidity) || isnan(temperature)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

Serial.print("Humidity: ");

Serial.print(humidity);

Serial.print(" %\t");

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" °C");

delay(2000);

}

Procedure

- Connect circuit

- Open Arduino IDE

- Install DHT library

- Select board & port

- Upload code

- Open Serial Monitor (9600 baud)

- Observe readings

Observation

| Parameter | Value |
| --- | --- |
| Temperature | ~24–25°C |
| Humidity | ~35–40% |

Conclusion

DHT11 sensor was successfully interfaced with Arduino, and temperature & humidity values were displayed, demonstrating sensor communication and real-time data acquisition.’

🔬 EXPERIMENT 3: Interfacing Seven Segment Display with Arduino

📄 Source:

Aim

To interface a seven-segment display with Arduino and display digits 0–9 sequentially.

Apparatus Required

- Arduino Uno

- Seven Segment Display (CA/CC)

- 220Ω resistors

- Breadboard

- Jumper wires

- USB cable

Theory

- A seven-segment display has 7 LEDs (a–g) + decimal point

- Used to display numbers (0–9)

Types

- Common Cathode (CC) → segments ON when HIGH

- Common Anode (CA) → segments ON when LOW

Pin Configuration (Example)

| Segment | Pin |
| --- | --- |
| a | 7 |
| b | 6 |
| c | 4 |
| d | 2 |
| e | 1 |
| f | 9 |
| g | 10 |

Common → GND (CC) / VCC (CA)

Algorithm

- Start

- Configure segment pins as OUTPUT

- Define patterns for digits (0–9)

- Display each digit

- Add delay

- Repeat

Code

unsigned const int A = 13;

unsigned const int B = 12;

unsigned const int C = 11;

unsigned const int D = 10;

unsigned const int E = 9;

unsigned const int F = 8;

unsigned const int G = 7;

unsigned const int H = 6;

void setup(void)

{

pinMode(A, OUTPUT);

pinMode(B, OUTPUT);

pinMode(C, OUTPUT);

pinMode(D, OUTPUT);

pinMode(E, OUTPUT);

pinMode(F, OUTPUT);

pinMode(G, OUTPUT);

pinMode(H, OUTPUT);

}

void zero(void) {

digitalWrite(A, LOW);

digitalWrite(B, HIGH);

digitalWrite(C, HIGH);

digitalWrite(D, HIGH);

digitalWrite(E, HIGH);

digitalWrite(F, HIGH);

digitalWrite(G, HIGH);

digitalWrite(H, LOW);

}

void one(void) {

digitalWrite(A, LOW);

digitalWrite(B, LOW);

digitalWrite(C, LOW);

digitalWrite(D, HIGH);

digitalWrite(E, LOW);

digitalWrite(F, LOW);

digitalWrite(G, HIGH);

digitalWrite(H, LOW);

}

void two(void) {

digitalWrite(A, HIGH);

digitalWrite(B, LOW);

digitalWrite(C, HIGH);

digitalWrite(D, HIGH);

digitalWrite(E, HIGH);

digitalWrite(F, HIGH);

digitalWrite(G, LOW);

digitalWrite(H, LOW);

}

void three(void) {

digitalWrite(A, HIGH);

digitalWrite(B, LOW);

digitalWrite(C, HIGH);

digitalWrite(D, HIGH);

digitalWrite(E, LOW);

digitalWrite(F, HIGH);

digitalWrite(G, HIGH);

digitalWrite(H, LOW);

}

void four(void) {

digitalWrite(A, HIGH);

digitalWrite(B, HIGH);

digitalWrite(C, LOW);

digitalWrite(D, HIGH);

digitalWrite(E, LOW);

digitalWrite(F, LOW);

digitalWrite(G, HIGH);

digitalWrite(H, LOW);

}

void five(void) {

digitalWrite(A, HIGH);

digitalWrite(B, HIGH);

digitalWrite(C, HIGH);

digitalWrite(D, LOW);

digitalWrite(E, LOW);

digitalWrite(F, HIGH);

digitalWrite(G, HIGH);

digitalWrite(H, LOW);

}

void six(void) {

digitalWrite(A, HIGH);

digitalWrite(B, HIGH);

digitalWrite(C, HIGH);

digitalWrite(D, LOW);

digitalWrite(E, HIGH);

digitalWrite(F, HIGH);

digitalWrite(G, HIGH);

digitalWrite(H, LOW);

}

void seven(void) {

digitalWrite(A, LOW);

digitalWrite(B, LOW);

digitalWrite(C, HIGH);

digitalWrite(D, HIGH);

digitalWrite(E, LOW);

digitalWrite(F, LOW);

digitalWrite(G, HIGH);

digitalWrite(H, LOW);

}

void eight(void) {

digitalWrite(A, HIGH);

digitalWrite(B, HIGH);

digitalWrite(C, HIGH);

digitalWrite(D, HIGH);

digitalWrite(E, HIGH);

digitalWrite(F, HIGH);

digitalWrite(G, HIGH);

digitalWrite(H, LOW);

}

void nine(void) {

digitalWrite(A, HIGH);

digitalWrite(B, HIGH);

digitalWrite(C, HIGH);

digitalWrite(D, HIGH);

digitalWrite(E, LOW);

digitalWrite(F, HIGH);

digitalWrite(G, HIGH);

digitalWrite(H, LOW);

}

void loop(void)

{

zero();

delay(100);

one();

delay(100);

two();

delay(100);

three();

delay(100);

four();

delay(100);

five();

delay(100);

six();

delay(100);

seven();

delay(100);

eight();

delay(100);

nine();

delay(100);

}

Procedure

- Connect segments via resistors

- Identify CA/CC type

- Upload code

- Observe digits

Observation

| Digit | Segments ON |
| --- | --- |
| 0 | a b c d e f |
| 1 | b c |
| 2 | a b d e g |
| 3 | a b c d g |
| 4 | b c f g |
| 5 | a c d f g |
| 6 | a c d e f g |
| 7 | a b c |
| 8 | a b c d e f g |
| 9 | a b c d f g |

Conclusion

Seven-segment display successfully interfaced and digits 0–9 displayed, demonstrating control of multiple output pins.

🔬 EXPERIMENT 4: Interfacing LCD 16×2 using I2C with Arduino

📄 Source:

Aim

To interface a 16×2 LCD display using I2C module with Arduino and display text.

Apparatus Required

- Arduino Uno

- 16×2 LCD with I2C module

- Breadboard

- Connecting wires

- USB cable

Theory

LCD Basics

- LCD displays characters using liquid crystals

- Controlled by internal controller

I2C Communication

- Uses only 2 lines:

  - SDA (Data)

  - SCL (Clock)

- Arduino acts as master

- LCD acts as slave

👉 Reduces wiring compared to parallel LCD

Connections

| LCD Pin | Arduino |
| --- | --- |
| VCC | 5V |
| GND | GND |
| SDA | A4 |
| SCL | A5 |

Algorithm

- Start

- Initialize LCD

- Turn ON backlight

- Set cursor position

- Display text

- Repeat

Program Code

#include <LiquidCrystal_I2C.h>

LiquidCrystal_I2C lcd(0x20,16,2);

void setup()

{

lcd.init();

lcd.clear();

lcd.backlight();      // Make sure backlight is on

lcd.setCursor(2,0);   //Set cursor to character 2 on line 0

lcd.print("TE-ECS");

lcd.setCursor(2,1);   //Move cursor to character 2 on line 1

lcd.print("GG Guys");

}

void loop()

{

}

Circuit

Agar Code nai chalra to library and boards check karna

Procedure

- Connect LCD via I2C

- Install LiquidCrystal_I2C library

- Upload code

- Observe display

Conclusion

LCD was successfully interfaced using I2C, reducing wiring complexity and demonstrating serial communication in embedded systems.

🔬 EXPERIMENT 5 (MODIFIED): Ultrasonic Sensor with STM32 using Arduino IDE

(Same hardware, just ditching CubeIDE drama)

Aim

To interface and program HC-SR04 Ultrasonic Sensor with STM32 using Arduino IDE to measure distance.

Apparatus Required

- STM32 (Blue Pill / similar)

- HC-SR04 Ultrasonic Sensor

- Breadboard

- Jumper wires

- USB / ST-Link (for upload)

- Arduino IDE

Theory (Same — no need to reinvent physics)

- Ultrasonic sensor sends sound waves

- Measures echo return time

- Calculates distance

Distance=Time×0.03432

Connections (Same Hardware)

| Sensor | STM32 |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TRIG | PA1 |
| ECHO | PA2 |

⚠ If your board isn’t 5V tolerant → use voltage divider on ECHO

Algorithm

- Set TRIG as OUTPUT

- Set ECHO as INPUT

- Send 10 µs pulse

- Measure echo duration

- Convert to distance

- Print on Serial Monitor

Circuit

I don’t know stm32 exps just pray you wont get one

Code

#define TRIG PA1

#define ECHO PA2

long duration;

float distance;

void setup() {

pinMode(TRIG, OUTPUT);

pinMode(ECHO, INPUT);

Serial.begin(9600);

}

void loop() {

// Clear trigger

digitalWrite(TRIG, LOW);

delayMicroseconds(2);

// Send pulse

digitalWrite(TRIG, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG, LOW);

// Read echo

duration = pulseIn(ECHO, HIGH);

// Calculate distance

distance = (duration * 0.0343) / 2;

// Print output

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

delay(500);

}

Procedure (Modified for Arduino IDE)

- Install STM32 board package in Arduino IDE

- Select board (e.g., STM32F103C8)

- Select correct COM port

- Connect circuit

- Paste code

- Upload (via ST-Link or USB)

- Open Serial Monitor

Observation

- Distance displayed in Serial Monitor

- Values change with object movement

Expected Output

Distance: 12.4 cm
Distance: 25.1 cm
Distance: 8.7 cm

Conclusion

The ultrasonic sensor was successfully interfaced with STM32 using Arduino IDE, demonstrating simplified embedded programming while maintaining accurate distance measurement.

🔬 EXPERIMENT 6: Servo Motor Control using PWM (STM32)

📄 Source:

Aim

To interface and control the position of a servo motor using PWM signals generated by STM32.

Apparatus Required

- STM32 board (Nucleo/Blue Pill)

- SG90 Servo Motor

- Breadboard

- Jumper wires

- ST-Link / USB

- STM32CubeIDE / Arduino IDE

Theory

- Servo motor = precise position control device

- Controlled using PWM (Pulse Width Modulation)

PWM Basics

- Frequency ≈ 50 Hz

- Pulse width controls angle:

| Pulse Width | Angle |
| --- | --- |
| 1 ms | 0° |
| 1.5 ms | 90° |
| 2 ms | 180° |

STM32 timers generate PWM signals to control position

Connections

| Servo Wire | Connection |
| --- | --- |
| Red | 5V |
| Brown/Black | GND |
| Orange/Yellow | PWM Pin (PA0 / TIM2_CH1) |

Algorithm

- Initialize PWM

- Set frequency = 50 Hz

- Generate PWM signal

- Vary duty cycle

- Servo rotates accordingly

- Repeat

Program Code (Arduino IDE – STM32)

#define SERVO_PIN PA0

void setup() {

pinMode(SERVO_PIN, OUTPUT);

}

void setAngle(int angle) {

int pulseWidth = map(angle, 0, 180, 1000, 2000); // µs

digitalWrite(SERVO_PIN, HIGH);

delayMicroseconds(pulseWidth);

digitalWrite(SERVO_PIN, LOW);

delay(20 - pulseWidth / 1000); // maintain 20ms period

}

void loop() {

setAngle(0);

delay(1000);

setAngle(90);

delay(1000);

setAngle(180);

delay(1000);

}

Circuit:

Nai run hua to bolo Motor ki galti hai code sahih ai motor nai chalra

Procedure

- Connect servo to STM32

- Configure PWM pin

- Upload code

- Observe servo movement

Observation

- Servo rotates to:

  - 0°

  - 90°

  - 180°

Conclusion

Servo motor position successfully controlled using PWM signals generated by STM32, demonstrating precise motor control.

🔬 EXPERIMENT 7: Multiple Task Creation in FreeRTOS

📄 Source:

Aim

To create multiple tasks and understand task management feature of FreeRTOS.

Theory

RTOS Concept

- Only one task runs at a time

- Scheduler switches between tasks

- Gives illusion of multitasking

FreeRTOS

- Lightweight RTOS for microcontrollers

- Provides:

  - Task scheduling

  - Inter-task communication

  - Synchronization

Task

- A task = independent function

- Runs in loop

- Has its own stack

Algorithm

- Start

- Create multiple tasks

- Assign priorities

- Start scheduler

- Tasks execute in switching manner

Program Code (Basic Example)

#include <Arduino.h>

#include <STM32FreeRTOS.h>

#define LED_PIN PA5

// Task 1: LED Blinking

void TaskLED(void *pvParameters) {

pinMode(LED_PIN, OUTPUT);

while (1) {

digitalWrite(LED_PIN, HIGH);

vTaskDelay(pdMS_TO_TICKS(500));

digitalWrite(LED_PIN, LOW);

vTaskDelay(pdMS_TO_TICKS(500));

}

}

// Task 2: Serial Printing

void TaskSerial(void *pvParameters) {

while (1) {

Serial.println("FAhhhhhhhhh...");

vTaskDelay(pdMS_TO_TICKS(1000));

}

}

void setup() {

Serial.begin(115200);

// Create Task 1 (LED)

xTaskCreate(TaskLED, "LED Task", 128, NULL, 1, NULL);

// Create Task 2 (Serial)

xTaskCreate(TaskSerial, "Serial Task", 128, NULL, 1, NULL);

// Start FreeRTOS Scheduler

vTaskStartScheduler();

}

void loop() {

// Not used

}

Circuit? Bas stm connect karlo

Conclusion

Multiple tasks were successfully created and executed using FreeRTOS, demonstrating multitasking and scheduler operation.

🔬 EXPERIMENT 8: Task Scheduling in FreeRTOS

📄 Source:

Aim

To illustrate task scheduling feature of FreeRTOS using multiple tasks.

Theory (From Your File, Cleaned)

- Task = independent function

- Multiple tasks share CPU

- Scheduler decides:

  - which task runs

  - when it runs

  - based on priority & state

Task States

- Running

- Ready

- Blocked

- Suspended

Scheduling Concepts

- Priority-based scheduling

  - Higher priority → executes first

- Preemptive scheduling

  - High priority task interrupts low priority

- Time slicing

  - Same priority → CPU shared equally

Program Code (Your Experiment)

/ --- Task Scheduler Experiment: Nucleo-F303RE ---
// Definitions for Task 2 and Task 3
const int externalLed = PA10; // D2 header
const int buzzerPin = PB3;    // D3 header
void setup() {
  // Initialize Pins
  pinMode(LED_BUILTIN, OUTPUT);
  pinMode(externalLed, OUTPUT);
  pinMode(buzzerPin, OUTPUT);
 
  // Initialize Task 4: Serial Communication
  Serial.begin(9600);
  while (!Serial); // Ensure monitor is ready
  Serial.println("========================================");
  Serial.println("  STM32 TASK SCHEDULER EXPERIMENT      ");
  Serial.println("========================================");
}
void loop() {
  // --- TASK 1: Onboard LED Control ---
  Serial.print("Executing Task 1... ");
  digitalWrite(LED_BUILTIN, HIGH);
  delay(400);
  digitalWrite(LED_BUILTIN, LOW);
  Serial.println("[DONE: Onboard LED Blinked]");

  // --- TASK 2: External LED Control ---
  Serial.print("Executing Task 2... ");
  digitalWrite(externalLed, HIGH);
  delay(400);
  digitalWrite(externalLed, LOW);
  Serial.println("[DONE: External LED Blinked]");
  // --- TASK 3: Buzzer Sound Control ---
  Serial.print("Executing Task 3... ");
  digitalWrite(buzzerPin, HIGH);
  delay(150); // Short beep
  digitalWrite(buzzerPin, LOW);
  Serial.println("[DONE: Buzzer Beeped]");
  // --- TASK 4: Scheduler Status Report ---
  Serial.println(">> STATUS: Full Cycle Completed Successfully.");
  Serial.println("----------------------------------------");
  // Gap between cycles
  delay(1500);
}

| Component | STM32 Pin | Wire Color (approx) |
| --- | --- | --- |
| External LED | PA10(D2) | Green/Yellow |
| Buzzer | PB3(D3) | Dark wire |
| GND | GND | Red/Black wires |
| Onboard LED | PA5 | Built-in |

Procedure

- Connect LEDs and buzzer

- Upload code

- Observe sequential execution

Observation

- LED1 → LED2 → Buzzer

- Repeats continuously

- Looks like multitasking

Conclusion

Task scheduling behavior was demonstrated, showing how multiple tasks can be executed in sequence, simulating multitasking.

🔬 EXPERIMENT 9: Queue Management in FreeRTOS

📄 Source:

Aim

To illustrate queue management feature of FreeRTOS for inter-task communication.

Theory

- Queue = FIFO (First In First Out) structure

- Used for safe communication between tasks

- One task sends data → another receives

Key Features

- Thread-safe

- Blocking support (wait if empty/full)

- Supports multiple tasks

- ISR-safe functions available

Working Concept

- Sender task → sends data to queue

- Receiver task → reads data

- Ensures no data conflict

Program Code (Your Experiment)

#include <Arduino.h>

#include <STM32FreeRTOS.h>

// 1. Create a queue handle [cite: 31]

QueueHandle_t myQueue;

// --- SENDER TASK --- [cite: 32]

void senderTask(void *pvParameters) {

int count = 0; // [cite: 34]

while (1) {

count++; // [cite: 36]

// Send data to queue [cite: 37]

// Uses portMAX_DELAY to wait if queue is full [cite: 15, 38]

xQueueSend(myQueue, &count, portMAX_DELAY);

Serial.print("Sent: "); // [cite: 39]

Serial.println(count); // [cite: 40]

vTaskDelay(1000 / portTICK_PERIOD_MS); // 1 second delay [cite: 41]

}

}

// --- RECEIVER TASK --- [cite: 44]

void receiverTask(void *pvParameters) {

int receivedValue; // [cite: 46]

while (1) {

// Receive data from queue [cite: 48, 49]

if (xQueueReceive(myQueue, &receivedValue, portMAX_DELAY)) {

Serial.print("Received: "); // [cite: 50]

Serial.println(receivedValue); // [cite: 51]

}

}

}

void setup() {

Serial.begin(9600); // [cite: 56]

// 2. Initialize the Queue: 5 elements, each of int type [cite: 57, 58]

myQueue = xQueueCreate(5, sizeof(int));

if (myQueue == NULL) { // [cite: 59]

Serial.println("Queue creation failed!"); // [cite: 60]

while (1); // [cite: 61]

}

// 3. Create tasks [cite: 63, 64, 65]

xTaskCreate(senderTask, "Sender", 128, NULL, 1, NULL);

xTaskCreate(receiverTask, "Receiver", 128, NULL, 1, NULL);

// 4. Start the scheduler [cite: 66, 67]

vTaskStartScheduler();

}

void loop() {

// Empty: FreeRTOS handles everything [cite: 69, 70]

}

Circuit connect stm

And hope it works

Procedure

- Create queue using xQueueCreate()

- Create sender & receiver tasks

- Upload code

- Open Serial Monitor

- Observe data transfer

Observation

Sent: 1
Received: 1
Sent: 2
Received: 2
Sent: 3
Received: 3

Conclusion

Queue successfully enabled safe and synchronized data transfer between tasks, demonstrating inter-task communication in FreeRTOS.

🔬 EXPERIMENT 10: Binary & Counting Semaphore in FreeRTOS

📄 Source:

Aim

To illustrate the use of Binary and Counting Semaphores for task synchronization in FreeRTOS.

Theory

Semaphore

- Used for task synchronization & resource sharing

Binary Semaphore

- Value = 0 or 1

- Used for signaling

- Example:

  - Task waits

  - Another task/ISR signals

Counting Semaphore

- Value = 0 to N

- Used for multiple resources

- Allows limited number of tasks to access resource

Key Functions

- xSemaphoreCreateBinary()

- xSemaphoreCreateCounting()

- xSemaphoreTake() → acquire

- xSemaphoreGive() → release

Program Code

🔹 Part A: Binary Semaphore

#include <STM32FreeRTOS.h>

// --- GLOBAL DECLARATIONS ---

// These must be outside setup() so all functions can see them

SemaphoreHandle_t xBinarySemaphore;

// Function Prototypes (telling the compiler these exist)

void TaskProducer(void *pvParameters);

void TaskConsumer(void *pvParameters);

void setup() {

Serial.begin(9600);

while (!Serial);

pinMode(PA5, OUTPUT); // Onboard LED

Serial.println("--- FreeRTOS Binary Semaphore Experiment ---");

// 1. Create the Semaphore

xBinarySemaphore = xSemaphoreCreateBinary();

if (xBinarySemaphore != NULL) {

// 2. Create Tasks

xTaskCreate(TaskProducer, "Producer", 128, NULL, 2, NULL);

xTaskCreate(TaskConsumer, "Consumer", 128, NULL, 1, NULL);

// 3. Start Scheduler

vTaskStartScheduler();

}

}

void loop() {

// Should be empty!

}

// --- TASK 1: PRODUCER ---

void TaskProducer(void *pvParameters) {

for (;;) {

Serial.println("Producer: Giving Semaphore...");

xSemaphoreGive(xBinarySemaphore);

vTaskDelay(pdMS_TO_TICKS(2000)); // Wait 2 seconds

}

}

// --- TASK 2: CONSUMER ---

void TaskConsumer(void *pvParameters) {

for (;;) {

// Wait for the flag

if (xSemaphoreTake(xBinarySemaphore, portMAX_DELAY) == pdTRUE) {

Serial.println("Consumer: Semaphore Taken! LED ON.");

digitalWrite(PA5, HIGH);

vTaskDelay(pdMS_TO_TICKS(200));

digitalWrite(PA5, LOW);

}

}

}

🔹 Part B: Counting Semaphore

Ye 8th wala circuit hai lekin code change kie hai

Ya wo bhi kie nai pata

#include <STM32FreeRTOS.h>

// --- Definitions ---

const int externalLed = PA10; // D2

const int buzzerPin = PB3;    // D3

// Declare Counting Semaphore Handle

SemaphoreHandle_t xCountingSemaphore;

void TaskProducer(void *pvParameters);

void TaskConsumer(void *pvParameters);

void setup() {

Serial.begin(9600);

while (!Serial);

// Initialize Hardware

pinMode(PA5, OUTPUT);

pinMode(externalLed, OUTPUT);

pinMode(buzzerPin, OUTPUT);

Serial.println("--- Scheduler with Counting Semaphore ---");

// Create Semaphore: Max 3 tokens (one for each hardware task)

xCountingSemaphore = xSemaphoreCreateCounting(3, 0);

if (xCountingSemaphore != NULL) {

xTaskCreate(TaskProducer, "Producer", 128, NULL, 2, NULL);

xTaskCreate(TaskConsumer, "Consumer", 128, NULL, 1, NULL);

vTaskStartScheduler();

}

}

void loop() {}

// --- TASK 1: THE PRODUCER (Triggers the Sequence) ---

void TaskProducer(void *pvParameters) {

for (;;) {

Serial.println("\nProducer: Releasing 3 Tokens for the Hardware Cycle...");

// Give 3 tokens to signal that 3 tasks are ready to run

xSemaphoreGive(xCountingSemaphore);

xSemaphoreGive(xCountingSemaphore);

xSemaphoreGive(xCountingSemaphore);

vTaskDelay(pdMS_TO_TICKS(5000)); // Wait 5 seconds between cycles

}

}

void TaskConsumer(void *pvParameters) {

int taskCount = 0;

for (;;) {

if (xSemaphoreTake(xCountingSemaphore, portMAX_DELAY) == pdTRUE) {

taskCount++;

if (taskCount == 1) {

Serial.println("Consumer: Task 1 - Onboard LED");

digitalWrite(PA5, HIGH); vTaskDelay(pdMS_TO_TICKS(500)); digitalWrite(PA5, LOW);

}

else if (taskCount == 2) {

Serial.println("Consumer: Task 2 - External LED");

digitalWrite(externalLed, HIGH); vTaskDelay(pdMS_TO_TICKS(500)); digitalWrite(externalLed, LOW);

}

else if (taskCount == 3) {

Serial.println("Consumer: Task 3 - Buzzer Beep");

digitalWrite(buzzerPin, HIGH); vTaskDelay(pdMS_TO_TICKS(200)); digitalWrite(buzzerPin, LOW);

taskCount = 0; // Reset for the next cycle

Serial.println(">> Full Scheduler Cycle Complete!");

}

}

}

}

Procedure

- Create semaphore

- Create producer & consumer tasks

- Upload program

- Observe synchronization

Observation

- Binary:

  - Task waits → signal received → executes

- Counting:

  - Limited tasks execute based on count

Conclusion

Binary semaphore enables task signaling, while counting semaphore manages multiple resources and controls task execution efficiently.